RX Chain Gain & Noise Summary for 10.5 GHz PLFM Radar

This document summarizes the receive (RX) chain assumptions, noise figure, link budget to the IF, and the recommended IF gain staging for your 16‑element dielectric‑filled slotted waveguide array using ADTR1107 LNAs and four ADAR1000 beamforming ICs. Update any assumptions marked as such with your measured values.

1) System & Assumptions

• Operating frequency: 10.5 GHz (λ ≈ 28.57 mm)

• TX power at antenna: 160 W ≈ 52.0 dBm

• Target RCS: σ = 1.0 m²

• Max range: 20 km

• Antenna gains (assumed): Gt = Gr = 31 dBi (update if different)

• IF: centered at 120 MHz with 30 MHz bandwidth

• RX array topology: 16 channels total → 4 channels per ADAR1000 coherently combined on‑chip; the 4 ADAR outputs are then combined via a 4‑way combiner (EP4RKU+).

Key parts and figures (typical, at ~10.5 GHz unless noted)

• ADTR1107 (per channel, RX path): Gain ≈ +18 dB, NF ≈ 2.5 dB

• ADAR1000 (per‑path RX gain): ≈ +9 dB; on‑chip 4‑path coherent sum → signal +12 dB (noise +6 dB), net +6 dB SNR

• EP4RKU+ 4‑way combiner: ideal combining loss 6.0 dB + excess IL ≈ 1–2 dB (assume 1.5 dB → ~7.5 dB total)

• M3SWA2‑34DR+ SPDT: insertion loss ≈ 1.0 dB (X‑band)

• BPF: insertion loss = 0.5 dB (user value)

• LTC5552 mixer: conversion loss ≈ 9 dB @ LO ≈ 0 dBm; LO operating window about −6 dBm to +6 dBm

• AD8352 IF amplifier/VGA: ~3–25 dB usable gain, broadband, differential‑friendly for ADC drive

• AD9484 ADC: ~1.5 Vpp_diff full‑scale input; target wideband noise ~−40 dBFS for good quantization margin

• ADF4382 LO source: program output so that ~0 dBm arrives at the LTC5552 LO port (use fixed pad if needed)

2) Radar‑Equation Echo (for context)

Monostatic radar equation (received power at the antenna):

Pr = Pt · Gt · Gr · λ² · σ / ((4π)³ · R⁴)

Using the assumptions above → Pr ≈ -121.9 dBm at the antenna (sensitive to Gt/Gr).

3) Cascaded Gain & Noise to Mixer IF

Inside each ADAR1000, 4 RX paths are coherently combined (+12 dB to the signal, +6 dB to noise → +6 dB SNR). Then the 4 ADAR outputs are externally combined with a 4‑way combiner.

Total small‑signal gain to the mixer IF (array → IF):
  +18.0 dB (ADTR1107) + (9.0+12) dB (ADAR1000) − 7.5 dB (combiner) − 1.0 dB (switch) − 0.5 dB (BPF) − 9.0 dB (mixer CL)
= **21.0 dB**

Receiver noise figure to the mixer input is dominated by the first LNA; a practical working number is **≈ 3.0 dB NF** for the overall front‑end.

4) Noise at IF and IF Gain Needed

Thermal noise over 30 MHz: N₀=−174 dBm/Hz → N_ant ≈ -99.2 dBm.

Add NF ≈ 3.0 dB → N_rx,in ≈ -96.2 dBm (referred to antenna).

Refer to the mixer IF output (before op‑amps): N_IF,out ≈ -75.2 dBm.

For AD9484, target the wideband noise at about **−40 dBFS** to keep quantization noise ~10 dB lower.

Practical target at ADC input: ≈ **−45 dBm equivalent**. From **-75.2 dBm** at the mixer IF, the required **post‑mixer IF gain** is:

**IF gain needed ≈ 30.2 dB**

5) IF Staging Recommendation

• **Two‑stage plan:** Stage‑1 VGA ≈ **+15 dB** nominal (with ±6–10 dB AGC) → inter‑stage 3–6 dB pad/LPF → Stage‑2 ADC driver ≈ **+10 dB** fixed. Total ≈ **25 dB**.

• **Single‑stage option:** One AD8352 up to **~25 dB**; two stages add AGC/linearity margin for strong clutter.

• Keep ≥10 dB crest headroom at ADC; follow AD9484 input network & common‑mode recommendations; maintain short, symmetric diff routing.

6) LO Drive & Mixer Note

• Set LTC5552 LO ≈ **0 dBm**; this yields ~8–10 dB conversion loss and stable linearity.

• If ADF4382 outputs ≈ **10 dBm**, reduce in registers or add a **10 dB** pad so ~0 dBm lands at the mixer LO pin (also improves return loss).

7) Sensitivities / To Update

• **Combiner total loss:** Assumed ~7.5 dB (6 dB ideal + 1.5 dB excess). If your measured EP4RKU+ loss differs, update it and recompute.

• **Antenna gains (Gt/Gr):** Replace the 31 dBi assumption with your measured/simulated array gains; recompute the radar‑equation echo if desired.

• **Measured NF:** If measured NF ≠ 3 dB, re‑calculate N_IF,out and IF gain needed (same target: noise near −40 dBFS).

• **IF bandwidth:** Narrower BW reduces required IF gain by 10·log10(BW ratio).

8) RX Line BoM (key parts)

• 16× ADTR1107 (per‑element T/R module; RX gain/NF)

• 4× ADAR1000 (4‑channel beamformer, on‑chip coherent RX combining)

• 1× EP4RKU+ 4‑way 0° combiner (external combine of 4 ADAR outputs)

• 1× M3SWA2‑34DR+ SPDT RF switch

• 1× X‑band BPF (IL ≈ 0.5 dB)

• 1× LTC5552 mixer (IF ≈ 120 MHz)

• 1–2× AD8352 (IF VGA/driver) or 1× AD8352 + 1× ADA4937‑1/ADL5565

• 1× AD9484 ADC

• 1× ADF4382 LO source (program or pad to ~0 dBm at LO pin)

• Inter‑stage pads, anti‑alias IF LPF, matching, bias, decoupling, and layout GND via‑fencing.